# Enhancing Coverage and Localization in Multi-Altitude UAV Active SLAM Using Recurrent Proximal Policy Optimization

Anonymous Author(s)

Abstract

Active Simultaneous Localization and Mapping (SLAM) for Unmanned Aerial Vehicles (UAVs) requires jointly maximizing map coverage and managing localization uncertainty while deciding at which altitude to fly. We propose a reinforcement learning framework that optimizes both objectives together. The proposed CoordConv–Long Short Term Memory (LSTM) recurrent Proximal Policy Optimization (PPO) actor-critic method is trained on a lightweight 48×48×4 altitude grid environment. The method learns to vote horizontal displacement and altitude changes under a composite reward that combines per-step coverage gain, frontier direction progress, loop closure bonuses, altitude breadth incentives, and a novel covariance trace shaping term. Evaluated across three training seeds and fifty held out maps against six classical baselines and a Vanilla PPO learned baseline, the proposed policy delivers higher final coverage, balanced exploration of every altitude layer, and a roughly 2× faster time to 50% coverage, while matching the best baseline on localization uncertainty. The headline coverage of 91.90±1.72% is statistically significant against every classical baseline as well as the Vanilla PPO learned baseline, and ablations on the base recipe confirm that the altitude breadth bonus stabilizes per-altitude exploration while the loop closure reward primarily reduces localization uncertainty.

# Introduction

Autonomous exploration of unknown three-dimensional environments is foundational for UAVs deployed in warehouses, tunnels, disaster zones, and inspection tasks [1], [2]. The problem is doubly coupled: the agent must move to reveal free space while localizing accurately enough to register it into a consistent global map, which is the defining concern of active SLAM [3], [4].

Classical active SLAM pipelines combine a frontier based or information theoretic controller with a backend SLAM estimator. These methods inherit two structural limitations: they plan on a single 2D occupancy grid, leaving altitude as a manually scheduled hyperparameter; and handcrafted cost functions struggle to jointly trade off exploration breadth, loop closure opportunity, and localization uncertainty.

Deep Reinforcement Learning (DRL) offers an attractive alternative, enabling a neural policy to learn all three concerns end-to-end. Recent work has demonstrated compelling results on 2D grids [5], [6], [7] and dynamic obstacle avoidance [8], but altitude aware active SLAM policies remain underexplored, and the reward engineering needed to make covariance reduction trainable alongside exploration is underdocumented.

The contributions of this work are as follows:

- Altitude as a separate decision: 3D exploration is decomposed into horizontal movement and altitude change. Altitude is selected via a vote with a minimum dwell rule to prevent rapid layer switching, and altitude information is fed back into the observation so a single recurrent policy handles both decisions jointly.

- Position-aware network with early fusion and memory: A CoordConv led convolutional encoder encodes absolute map position, map features and SLAM scalars are combined via early fusion, and a recurrent layer carries context across the episode. This combination has not been applied to multi-altitude active SLAM before.

- Reward shaping for localization: A reward term Δσ on the SLAM uncertainty surrogate σt (a proxy for Tr(Σt)) densifies the rare loop closure signal, pushing the policy toward localization improving actions. A per-altitude breadth bonus further stabilizes layer allocation, as confirmed by ablations.

- Thorough evaluation: The policy is benchmarked against six classical baselines and a Vanilla PPO baseline across multiple seeds and held out maps, achieving higher final coverage, balanced per-altitude exploration, and faster half coverage time with statistically significant gains throughout.

# Related Work

Prior 3D coverage and localization work splits into classical information-theoretic active SLAM controllers and learned DRL exploration policies; we walk each thread, then surface the gap our framework closes.

Classical active SLAM in 3D: Placed et al. [3] survey frontier, information-theoretic, and sampling based exploration through 2023. Asgharivaskasi and Atanasov optimize Shannon mutual information over semantic 3D octomaps [9], extended to Riemannian multi robot optimization in [10]. Ebadi et al. [11] and Azpúrua et al. [12] review 3D SLAM and autonomous exploration in extreme/subterranean environments. These methods are interpretable but plan in continuous 3D space with handcrafted costs; they neither learn a policy nor decompose altitude into discrete decisions.

Learned 3D exploration policies: Chen et al. [6] present a LiDAR based end-to-end DRL active SLAM policy; Zhao and Hwang [7] propose an exploration–exploitation DDPG variant. Botteghi et al. [13] use curiosity driven rewards for indoor mapping; Yin et al.’s MA-SLAM [5] contributes a structured map representation for large scale DRL exploration on ground robots. Garaffa et al. [14] survey the broader literature; Wang et al. [15] address exploration enhanced source search. None is altitude-aware: existing 3D capable methods operate on full continuous 3D actions or a single planar slice.

RL for UAVs in 3D: NavRL [8] trains PPO for safe 3D UAV flight with sim-to-real transfer, but targets collision free goal reaching, not mapping. Kaufmann et al. [16] surpass human champions at 3D drone racing, which is again navigation rather than exploration. Ni et al. [17] establish recurrent model-free RL as a strong Partially Observable Markov Decision Process (POMDP) baseline (motivating our LSTM backbone); Raffin et al. [18] provide the Stable-Baselines3 Recurrent PPO used here.

# System Model and Problem Formulation

## System Model

Fig. 1 depicts the closed loop system for autonomous warehouse inspection. The full 3D planning problem is decomposed into (i) horizontal navigation within a fixed altitude layer and (ii) discrete altitude selection, with a single recurrent policy arbitrating between the two.

UAV and environment: A quadrotor carries a 2D LiDAR scanner performing a 360∘ sweep with Nr​=​36 beams at the current altitude, plus a half-range sweep on each adjacent layer to emulate inter-layer sensor bleed. LiDAR is preferred for its illumination robustness and native compatibility with occupancy mapping. The warehouse is partitioned into four altitude layers halt​∈​{1,2,3,4} m over an X​×​Y grid; the unknown true occupancy is a binary tensor M⋆​∈​{0,1}X×Y×4 to be reconstructed within a fixed episode budget.

Perception and observation: The full sweep {ri,t}i=1Nr is fused by an RTAB-Map inspired SLAM proxy maintaining an occupancy estimate Mt, pose estimate ξt, and uncertainty surrogate σt​∈​[0,4] (proxy for Tr(Σt)), updated as σt+1​=​min(σt+0.03 ∥Δpt∥+0.15 1[alt change], 4) and multiplied by 0.70 on loop closure. This is compressed into ot​=​(Mt,vt): the map tensor Mt​∈​[0,1]5×48×48 stacks five altitude-aware channels (current altitude occupancy; binary visit mask; trajectory trail; least explored altitude occupancy; cross altitude mean visit mask), and vt​∈​[0,1]10 encodes normalized uncertainty, frontier count, coverage, frontier distance and direction, altitude index, per-altitude coverage statistics, and step fraction t/Tmax.

Policy and control loop: The recurrent policy πθ(at​∣​ot,ht), parameterized by θ with recurrent hidden state ht (LSTM state, defined in Section 4), emits a continuous action at​=​(dxt,dyt,dzt)​∈​[−1,1]3: the first two components drive a clipped horizontal velocity command; dzt is an altitude vote that triggers a layer change only when |dzt|​>​τalt, where τalt​=​0.3 is the altitude vote threshold, and is further gated by a minimum dwell of D​=​20 steps since the last layer change, preventing the policy from thrashing the mapping backend’s Z-filter.

System model: 2D LiDAR feeds a SLAM back-end producing observation ot for the RL policy, which emits action at.

## Problem Formulation

We cast the task as a POMDP P​=​⟨S,A,O,T,Oobs,R,γ,Tmax⟩ over state, action, and observation spaces, with transition kernel T, observation kernel Oobs, reward R, discount γ, and horizon Tmax. The hidden state bundles the (true, unhatted) UAV pose, occupancy Mt, and the scalar uncertainty surrogate σt from the System Model, namely st=(xt,yt,zt,Mt,σt)∈S, with A​=​[−1,1]3 and O​=​[0,1]5×48×48​×​[0,1]10 inheriting the dimensions of at and ot​=​(Mt,vt) from the System Model; Oobs(⋅|st) realizes the SLAM to observation compression, and T(⋅|st,at) is driven by UAV kinematics, SLAM updates, and loop closure events. Heading ψt is not stored: it is implicit in the velocity command and recomputed on demand from (pt−pt−1) when needed for the relative frontier direction scalar.

Objective as a function of θ: Let πθ​:​O​×​H​→​Δ(A) be the recurrent policy parameterized by θ, with hidden state space H (the LSTM state of Section 4) and Δ(A) the simplex of distributions over A; let τ​=​(s0,o0,a0,…,sTmax) be a trajectory it induces on P. The expected discounted return is

J(θ) = Eτ∼πθ​ t=0Tmax−1γ t R(st,at) ,

with R the composite reward of Section 4.2; γ​=​0.995 gives an effective horizon 1/(1−γ)​≈​200 steps, matched to the Tmax​=​600-step budget of the headline recipe (∼150 steps per altitude over four layers). The target is θ⋆=argmaxθJ(θ) with π⋆=πθ⋆, subject to the constraint set  C: at∈[−1,1]3,∀ tat∼πθ(⋅​∣​ot,ht),∀ tst+1∼T(⋅​∣​st,at),∀ tot∼Oobs(⋅​∣​st),∀ talt(t)∈{0,1,2,3},∀ tt−τlast(t)≥D=20,if alt. change at t(xt,yt,zt)∈F,∀ t0≤t<Tmax,Tmax​∈​{400,600}Cov(st)​≥​0.95⇒terminate,

where alt(t)​=​⌊zt⌋​−​1​∈​{0,1,2,3} indexes the four altitude layers (recall zt​∈​{1,2,3,4} m), F is collision free configuration space, τlast(t) is the most recent altitude change timestep, and D​=​20 is the dwell from the System Model (set to suppress Z-filter thrashing). For ablation variants and the base/Vanilla recipes the horizon is Tmax​=​400; only the final headline “MS600” recipe uses Tmax​=​600.

Coverage–localization trade-off: Let Cov(s) be the fraction of true free cells in M⋆ that have been observed by state s, and define K(θ)​:=​Eτ∼πθ[Cov(sTmax)], U(θ)​:=​Eτ∼πθ[σTmax]. Equation [eq:J] then admits the scalarized form J(θ)=K(θ)−λσ U(θ)+Ψ(θ) (coverage maximization, uncertainty minimization, and shaping, respectively), with λσ​>​0 realized implicitly through the reward weights and Ψ(θ) collecting the per-step shaping terms (new cell, frontier, loop closure, breadth, absolute σ penalty, and Δσ) of Section 4.2.

# Proposed Methodology

The policy πθ is a CoordConv–LSTM actor-critic trained with Recurrent PPO [17], [18]. It is shaped by three task specific requirements: (i) the filters must exploit absolute spatial position on the altitude grid; (ii) the spatial map tensor and the scalar feature vector must be fused before the policy commits to an action; and (iii) partial observability demands memory across the 600-step episode. Fig. 2 shows the complete network.

Proposed CoordConv–LSTM actor-critic. Map tensor is encoded by one CoordConv and three Conv2D stages; scalars by an MLP. Early fused 256-D embedding feeds a single layer LSTM (hidden state ht, dashed self-loop), then two (256,256) heads output the action distribution and state value.

## Network Design

Spatial map encoder: Starting from the five altitude aware channels of Mt​∈​[0,1]5×48×48 (current altitude occupancy, current altitude binary visit mask, exponentially decaying trajectory, least explored altitude occupancy as a soft exploration prior, and cross altitude mean visit mask), one CoordConv (kernel 5, stride 2) and three plain Conv2D stages (two stride-2 followed by one stride-1, all kernel 3) compute

Ft(ℓ) = σGN8W(ℓ)​*​Ft(ℓ−1)+b(ℓ),

with Ft(0)​=​Mt, weights W(ℓ) and biases b(ℓ), channel widths 32​→​64​→​64​→​64 across the four stages, σ(⋅) the ReLU, and GN8 a GroupNorm with 8 groups. Only the first stage is coordinate augmented: Ft(0) stacks Mt with two normalized coordinate maps (i/H, j/W) over feature map indices (i,j) of height H and width W; for ℓ​≥​1, Ft(ℓ−1)​=​Ft(ℓ−1). These coordinate channels give filters direct access to absolute position, a property plain convolutions lack and one that matters in active SLAM, where the same local pattern has different navigational value at a map’s centre than at its boundary. The three strided stages collapse the 48×48 input through 24×24​→​12×12​→​6×6; the fourth (stride-1) stage refines features at 6×6, and an adaptive 6×6 average pool AvgPool6 (acting as identity here) followed by flattening yields the map feature ztmap=vec(AvgPool6(Ft(4)))∈R2304.

Scalar encoder and early fusion: The vector vt​∈​[0,1]10 (normalized covariance trace, frontier statistics, per-altitude coverages, episode progress) is layer normalized and passed through a two layer MLP gψv with parameters ψv, giving ztscl​=​gψv(vt)​∈​R32. The map and scalar embeddings are concatenated and reprojected by a two layer fusion MLP gψf with parameters ψf, giving zt=gψf([ztmap;ztscl])∈R256. This early fusion is deliberate: the policy must reason over spatial layout and scalar state jointly (“move toward this frontier because covariance is high”), whereas late fusion would force the branches to commit to independent sub-decisions before merging.

Recurrent backbone: A single layer LSTM with hidden size 128 and parameters ϕ propagates context across the episode, (ht,ct)=LSTMϕ(zt;ht−1,ct−1) with ht,ct​∈​R128, where ht is the hidden state, ct the cell state, and (h0,c0)​=​0 at episode start. Recurrence is indispensable: the observation is a lossy compression of history (the trajectory channel fades old visits, and vt squeezes the SLAM state into ten numbers), so ht must carry intent across multi-step strategies (e.g. “commit to altitude 3 for 40 steps”) that no Markovian head could express.

Actor and critic heads: Two independent (256,256) MLPs fθπ,fθV with parameters θπ,θV branch from ht. The actor parameterizes a diagonal Gaussian over R3 with mean μt​∈​R3 and log standard deviation logσta​∈​R3: (μt,logσta)=fθπ(ht) and at∼N(μt,diag((σta)2)), where the action standard deviation σta is distinct from the uncertainty surrogate σt; the critic outputs a scalar state value VθV(ot)​=​fθV(ht). Training minimizes the joint objective

L(θπ,θV) = LCLIP(θπ) + c1 (VθV(ot)​−​Rt)2 − c2 H[πθπ],

where LCLIP is PPO’s clipped surrogate objective, Rt is the Generalized Advantage Estimation (GAE) target, H[πθπ] an action-entropy bonus, and c1,c2​>​0 are scalar weighting coefficients [18]. Separate actor/critic parameters let the critic fit the higher-frequency value signal without destabilizing the actor.

## Reward Design

The per-step reward rt is a weighted sum of fifteen components (Table 1):

rt=wΔn Δncell+wΔd Δdfront+wσ (σt/4)+wΔσ Δσ+walt 1[new alt]+wbr 1[breadth]+wstag 1[stag]+rmisc,

with terminal bonuses at episode end. Three design decisions are highlighted.

Exploration vs. localization: Δncell and Δdfront reward exploration directly. Localization is shaped by an absolute penalty σt/4 and the delta term Δσ (wΔσ​=​−0.8), which densifies the sparse loop closure event (σ cut by 0.70) into a per-step signal, biasing the agent toward uncertainty reducing actions without dominating exploration terms.

Altitude breadth: A one time new altitude bonus (+25) incentivizes layer visits; the breadth bonus (+60 per altitude on first reaching 40% coverage) caps at 4​×​60​=​240 and backloads credit onto neglected layers. Removing it drops mean coverage by 1.0 pt and triples cross-seed variance on Alt0/Alt3 (Section 5.2).

Stagnation and terminal shaping: A stagnation penalty (−15, when per-altitude gain <​2% over 25 steps) and terminal penalty (−120​⋅​σT/4) prevent local minima convergence. The terminal coverage bonus is breadth weighted:

rterminal=300⋅Cov⋅(0.5+0.5 ρ),

where ρ is the fraction of altitudes crossing the 40% threshold, halving terminal credit if any layer is neglected.

Reward components (1[⋅]: indicator). Δσ is our contribution (MS600 only); Vanilla PPO keeps only Δncell, per-step, and collision.

| Term | Wt. | Term | Wt. |
| --- | --- | --- | --- |
| Δncell (new cells) | +1.0​ | 1[stag.] (stagnation) | −15.0​ |
| Δdfront (frontier) | +2.0​ | 1[cov​≥​0.4]​ | +30.0​ |
| 1[coll.] (collision) | −1.0​ | 1[LC] (loop closure) | +25.0​ |
| per-step (budget) | −0.05​ | 1[breadtha] (×4) | +60.0​ |
| revisit (7×7) | −0.15​ | σt/4 (abs. unc.) | −0.8​ |
| 1[new alt]​ | +25.0​ | Δσ (ours) | −0.8​ |
| 1[alt sw]​ | −3.0​ | Terminal cov. (×Cov(0.5+0.5ρ)) | +300​ |
| 1[alt sw bad]​ | −2.0​ | Terminal σ (×σT/4) | −120​ |
| 1[least-expl.]​ | +0.5​ |  |  |

# Performance Evaluation

## Experimental Conditions

### Training Configuration

Three independent seeds {42,100,200} share an identical Recurrent PPO setup: linear learning rate schedule 3×10−4​→​10−5, entropy coefficient 0.02​→​0.002, 4 parallel envs, nsteps=256, batch 128, nepochs=5, γ=0.995, GAE λ=0.97, clip 0.2, max-grad-norm 0.5, and reward-only normalization. Each seed trains for 800,000 steps with checkpoints every 60,000 steps, under a three phase easy→medium (120k)→hard (300k) difficulty curriculum.

### Checkpoint Selection

We select the earliest checkpoint that simultaneously (i) exceeds the strongest classical baseline (Greedy Information Gain) by at least 3 percentage points of coverage, (ii) achieves σT no greater than 0.80​×​ Greedy’s, and (iii) keeps every per-altitude coverage above 25%. For the MS600 recipe this rule is satisfied earliest at step 180,000 on every seed, and that checkpoint is fixed for all downstream evaluation (with a composite-score fallback if no checkpoint satisfies all three conditions on a seed).

### Baselines and Evaluation Protocol

Six classical non-learned baselines serve as comparators: Random Walk, Nearest Frontier, Spiral, Potential Field, Greedy Information Gain [3], and an RRT style frontier explorer in the spirit of Umari and Mukhopadhyay [3]. A Vanilla PPO learned baseline shares our network, hyperparameters, and curriculum, but disables every shaping reward except Δncell, the per-step cost, and the collision penalty. Every method is evaluated on the same 50 held-out map seeds with identical initial conditions; RL variants are rolled out per training seed (n​=​150 episodes total). All evaluation runs use Tmax​=​600 for MS600, Tmax​=​400 otherwise, and significance is assessed via Wilcoxon signed-rank tests with Cliff’s δ as the effect size.

## Results

### Main Comparison

Table [tab:main] reports all twelve methods (six classical baselines, Vanilla PPO, three learned recipe variants, and two ablations). Our headline recipe reaches 91.90​±​1.72% coverage, a 9.17-point absolute gain over the best classical baseline (Greedy Information Gain at 82.73%) and 5.03 points over Vanilla PPO. The per-altitude breakdown shows the breadth-reward effect: our policy exceeds 85% on every altitude, while Greedy Info Gain collapses to 69.93% on Alt3 and RRT Explorer to 9.48%.

### Statistical Analysis

Table 2 pools Wilcoxon signed-rank tests of the headline MS600 policy across seeds (n=150). Our method rejects the null at p​<​10−13 with δ​>​0.97 (“large” by Cliff’s convention) against every non-learned baseline except Greedy Information Gain. The gap against Greedy Information Gain (p​=​8.1×10−14, δ​=​+0.534) is now itself a large effect; the gap against Vanilla PPO is p​=​7.4×10−11 with δ​=​+0.486 (large), quantifying the reward design’s contribution over bare RL with identical architecture and curriculum.

One-sided Wilcoxon tests on coverage, MS600 vs. each baseline, pooled n=150. * p​<​0.05, ** p​<​0.01, *** p​<​0.001.

| Comparison | p-value | Cliff’s δ | Effect |
| --- | --- | --- | --- |
| Ours vs. Random Walk | 1.2×10−26*** | +0.990​ | large |
| Ours vs. Nearest Frontier | 1.4×10−26*** | +0.979​ | large |
| Ours vs. Spiral | 1.2×10−26*** | +1.000​ | large |
| Ours vs. Potential Field | 1.2×10−26*** | +0.974​ | large |
| Ours vs. Greedy Info Gain | 8.1×10−14*** | +0.534​ | large |
| Ours vs. RRT Explorer | 1.2×10−26*** | +0.997​ | large |
| Ours vs. Vanilla PPO | 7.4×10−11*** | +0.486​ | large |

### Training and Intra-Episode Dynamics

(a) Coverage vs. training steps (mean ± std, 3 seeds; dashed: Greedy Info Gain reference). (b) Per-altitude learning curves (solid: RL; dotted: Greedy final altitude coverage). (c) Intra-episode coverage vs. step over 50 held-out episodes; RL reaches 50% at step ∼​180, ∼​2× faster than Greedy.

Fig. 3(a) shows that training coverage rises monotonically and exceeds the Greedy baseline by step 120,000. Fig. 3(b) reveals the multi-altitude strategy: the policy saturates the middle layers (Alt1, Alt2, where shelf content is densest) first, then brings the boundary layers (Alt0, Alt3) above 85%, a regime no classical baseline reaches. Fig. 3(c) shows per-step coverage across 50 evaluation episodes: the RL policy is both more thorough (higher final coverage) and faster per unit time, reaching the 50%-coverage milestone at step ∼​180 (RL) vs. ∼​370 (Greedy) vs. ∼​580 (Nearest Frontier).

### Qualitative Multi-Altitude Trajectory

Representative learned trajectory on a held-out map (seed 221; final coverage 95.1%; one grid cell =1 m). Each panel is one altitude layer, showing obstacles (grey) and the agent’s flight path while at that layer, colour-coded by episode step, with enter (circle) and exit (square) markers. The policy allocates comparable effort to all four layers.

Fig. 4 visualizes a representative rollout across the four altitude layers. The policy distributes its dwell almost evenly across layers (136, 124, 106, and 112 steps) while reaching 95.1% coverage: each layer is swept densely, the policy then changes altitude (respecting the dwell rule), and revisits near previously mapped regions generate the loop closures that keep σt bounded, in contrast to classical planners, which collapse onto the boundary layers (Table [tab:main]).

### Ablation Analysis

Two ablations are run on the base recipe (Tmax​=​400, no Δσ shaping; mean coverage 87.56​±​2.45%) across the same three seeds. No LoopClosure (89.41​±​2.31%): mean coverage is statistically indistinguishable from base (Δ​=​+1.85 pts), but σT rises from 3.71 to 3.82, confirming that loop closure primarily reduces localization uncertainty rather than improving coverage. No BreadthBonus (86.54​±​1.49%, −1.02 pts vs. base): the mean drop understates the effect; cross-seed standard deviations on Alt0 and Alt3 jump from 4.97% and 4.83% to 9.85% and 9.38%, showing that the breadth bonus stabilizes per-altitude allocation across seeds rather than raising the mean. Together with the +3.21-pt MS600 horizon extension, these components account for the full 91.90% headline.

# Computational Cost and Real-Time Feasibility

A policy intended for onboard UAV deployment must fit a companion computer’s compute and latency budget. The network has 2,023,035 trainable parameters (≈​2.02 M) and requires 10.16 M multiply–accumulate operations (20.32 M FLOPs) per inference, occupying ≈​8 MB at single precision, well within embedded-class memory. Measured on a single thread (OMP/MKL pinned) of an Intel Core i5-12450H with no GPU, emulating a UAV companion computer, the forward pass completes in 1.20 ms on average (1.55 ms at the 99th percentile, ∼​836 Hz), and the full control step including observation construction takes 1.98 ms (∼​506 Hz). Even a conservative 10–20× slowdown on an ARM-class processor sustains 25–80 Hz, far above the ∼​0.5 Hz waypoint-decision rate. Inference is therefore not a deployment bottleneck; the binding real-time constraint is the SLAM backend, discussed next.

# Sim-to-Real Transfer: Findings and Roadmap

The lightweight grid environment mirrors a high-fidelity simulation stack to ease eventual hardware transfer. A companion Gazebo/PX4 software-in-the-loop simulation hosts an x500 quadrotor with a 360∘ 2D LiDAR and an RGB-D camera, and RTAB-Map performs real-time visual SLAM, producing the occupancy grid, covariance, and loop closures the environment abstracts. Because the observation and action interfaces match the training environment exactly (a five-channel 48​×​48 map tensor, a ten-dimensional scalar vector, and a three-dimensional action), the trained checkpoint runs zero-shot in Gazebo without fine-tuning. A reactive safety layer (LiDAR/depth avoidance with a covariance-triggered retreat) and a three-tier diagnostic are operational; in an altitude-capped 300-step run the policy explores with stable odometry and zero tracking-loss events, validating the end-to-end pipeline on a real SLAM backend.

The dominant gap between the abstract grid and physical SLAM is motion smoothness: RTAB-Map’s visual odometry requires smooth motion, and abrupt reversals, which the grid agent can issue freely because it teleports between cells, degrade tracking and can cascade into pose drift. The deployed, no-retraining mitigation is a per-axis exponential-moving-average action filter that preserves sustained sweeps while damping jitter. Building on this, the environment exposes three transfer-oriented controls (disabled in the experiments above, so the reported results are unaffected): a motion-smoothness reward −wsm∥at−at−1∥2 that reduces reliance on inference-time filtering, an altitude-transition penalty that discourages the Z-filter thrashing which stalls RTAB-Map re-slicing, and a frontier definition aligned to the RTAB-Map projection. Further milestones include altitude-dependent feature-density modelling, larger geometries, and on-board flight tests. Because the policy already optimizes a localization-uncertainty surrogate, it is predisposed toward the loop-closure-seeking behaviour that real SLAM backends reward.

# Conclusion and Future Work

We presented a Recurrent PPO approach to multi altitude UAV active SLAM, in which a CoordConv–LSTM actor-critic jointly controls horizontal motion and altitude selection. A fifteen term composite reward, including a novel Δσ uncertainty-shaping term, a per-altitude breadth bonus, and breadth weighted terminal credit, produces policies reaching 91.90​±​1.72% coverage across three seeds and fifty held-out maps, dominating all baselines by large effect sizes (Cliff’s δ​≥​0.49; Table 2). Ablations confirm the breadth bonus stabilizes per-altitude allocation and the loop closure reward primarily reduces localization uncertainty. Future work will scale to larger geometries, fold a real RTAB-Map backend into the training loop, and pursue physical UAV deployment following the roadmap of Section 7.

18

Qin, H., Meng, Z., Meng, W., Chen, X., Sun, H., Lin, F., Ang, M.H.: Autonomous exploration and mapping system using heterogeneous uavs and ugvs in gps-denied environments. IEEE Transactions on Vehicular Technology 68(2), 1339–1350 (2019). https://doi.org/10.1109/TVT.2018.2890416

Zhang, Y., Tan, Z., Liu, K., Yang, Y., Yang, W., Wang, S., Wen, C.Y.: Adaptive clustering convexification mapping-planning system for uavs in unknown environments. IEEE Transactions on Automation Science and Engineering 23, 5852–5868 (2026). https://doi.org/10.1109/TASE.2026.3670813

Placed, J.A., Strader, J., Carrillo, H., Atanasov, N., Indelman, V., Carlone, L., Castellanos, J.A.: A survey on active simultaneous localization and mapping: State of the art and new frontiers. IEEE Transactions on Robotics 39(3), 1686–1705 (2023). https://doi.org/10.1109/TRO.2023.3248510

Zhang, J., Qian, H., Wang, Y., Liang, C., Song, A.: U-slam: Ultrasonic simultaneous localization and mapping for underwater pipeline defect inspection. IEEE Transactions on Instrumentation and Measurement 75, 1–14 (2026). https://doi.org/10.1109/TIM.2026.3674248

Yin, Y., Qi, Y., Feng, D., Chen, H., Ma, H., Wu, J., Jiang, Y.: MA-SLAM: Active SLAM in large-scale unknown environment using map aware deep reinforcement learning. arXiv preprint arXiv:2511.14330 (2025). https://doi.org/10.48550/arXiv.2511.14330

Chen, J., Wu, K., Hu, M., Suganthan, P.N., Makur, A.: LiDAR-based end-to-end active SLAM using deep reinforcement learning in large-scale environments. IEEE Transactions on Vehicular Technology 73(10), 14187–14200 (2024). https://doi.org/10.1109/TVT.2024.3405483

Zhao, S., Hwang, S.H.: Exploration- and exploitation-driven deep deterministic policy gradient for active SLAM in unknown indoor environments. Electronics 13(5),  999 (2024). https://doi.org/10.3390/electronics13050999

Xu, Z., Han, X., Shen, H., Jin, H., Shimada, K.: NavRL: Learning safe flight in dynamic environments. IEEE Robotics and Automation Letters 10(4), 3668–3675 (2025). https://doi.org/10.1109/LRA.2025.3546069

Asgharivaskasi, A., Atanasov, N.: Semantic OcTree mapping and shannon mutual information computation for robot exploration. IEEE Transactions on Robotics 39(3), 1910–1928 (2023). https://doi.org/10.1109/TRO.2023.3245986

Asgharivaskasi, A., Girke, F., Atanasov, N.: Riemannian optimization for active mapping with robot teams. IEEE Transactions on Robotics 41, 1077–1097 (2025). https://doi.org/10.1109/TRO.2025.3526295

Ebadi, K., Bernreiter, L., Biggie, H., Catt, G., Chang, Y., Chatterjee, A., Denniston, C.E., Deschênes, S.P., Harlow, K., Khattak, S., et al.: Present and future of SLAM in extreme environments: The DARPA SubT challenge. IEEE Transactions on Robotics 40, 936–959 (2024). https://doi.org/10.1109/TRO.2023.3323938

Azpúrua, H., Saboia, M., Freitas, G.M., Clark, L., Agha-mohammadi, A.a., Pessin, G., Campos, M.F.M., Macharet, D.G.: A survey on the autonomous exploration of confined subterranean spaces: Perspectives from real-world and industrial robotic deployments. Robotics and Autonomous Systems 160, 104304 (2023). https://doi.org/10.1016/j.robot.2022.104304

Botteghi, N., Sirmacek, B., Poel, M., Brune, C., Schulte, R.: Curiosity-driven reinforcement learning agent for mapping unknown indoor environments. ISPRS Annals of the Photogrammetry, Remote Sensing and Spatial Information Sciences V-1-2021, 129–136 (2021). https://doi.org/10.5194/isprs-annals-V-1-2021-129-2021

Garaffa, L.C., Basso, M., Konzen, A.A., de Freitas, E.P.: Reinforcement learning for mobile robotics exploration: A survey. IEEE Transactions on Neural Networks and Learning Systems 34(8), 3796–3810 (2023). https://doi.org/10.1109/TNNLS.2021.3124466

Wang, M., Xin, B., Jing, M., Qu, Y.: An exploration-enhanced search algorithm for robot indoor source searching. IEEE Transactions on Robotics 40, 4160–4178 (2024). https://doi.org/10.1109/TRO.2024.3454572

Kaufmann, E., Bauersfeld, L., Loquercio, A., Müller, M., Koltun, V., Scaramuzza, D.: Champion-level drone racing using deep reinforcement learning. Nature 620(7976), 982–987 (2023). https://doi.org/10.1038/s41586-023-06419-4

Ni, T., Eysenbach, B., Salakhutdinov, R.: Recurrent model-free RL can be a strong baseline for many POMDPs. In: Proceedings of the 39th International Conference on Machine Learning (ICML). Proceedings of Machine Learning Research, vol. 162, pp. 16691–16723. PMLR (2022)

Raffin, A., Hill, A., Gleave, A., Kanervisto, A., Ernestus, M., Dormann, N.: Stable-Baselines3: Reliable reinforcement learning implementations. Journal of Machine Learning Research 22(268),  1–8 (2021)

[1] 	H. Qin et al., “Autonomous exploration and mapping system using heterogeneous UAVs and UGVs in GPS-denied environments,” IEEE Transactions on Vehicular Technology, vol. 68, no. 2, pp. 1339–1350, 2019, doi: 10.1109/TVT.2018.2890416.

[2] 	Y. Zhang et al., “Adaptive clustering convexification mapping-planning system for UAVs in unknown environments,” IEEE Transactions on Automation Science and Engineering, vol. 23, pp. 5852–5868, 2026, doi: 10.1109/TASE.2026.3670813.

[3] 	J. A. Placed et al., “A survey on active simultaneous localization and mapping: State of the art and new frontiers,” IEEE Transactions on Robotics, vol. 39, no. 3, pp. 1686–1705, 2023, doi: 10.1109/TRO.2023.3248510.

[4] 	J. Zhang, H. Qian, Y. Wang, C. Liang, and A. Song, “U-SLAM: Ultrasonic simultaneous localization and mapping for underwater pipeline defect inspection,” IEEE Transactions on Instrumentation and Measurement, vol. 75, pp. 1–14, 2026, doi: 10.1109/TIM.2026.3674248.

[5] 	Y. Yin et al., “MA-SLAM: Active SLAM in large-scale unknown environment using map aware deep reinforcement learning,” arXiv preprint arXiv:2511.14330, 2025, doi: 10.48550/arXiv.2511.14330.

[6] 	J. Chen, K. Wu, M. Hu, P. N. Suganthan, and A. Makur, “LiDAR-based end-to-end active SLAM using deep reinforcement learning in large-scale environments,” IEEE Transactions on Vehicular Technology, vol. 73, no. 10, pp. 14187–14200, 2024, doi: 10.1109/TVT.2024.3405483.

[7] 	S. Zhao and S.-H. Hwang, “Exploration- and exploitation-driven deep deterministic policy gradient for active SLAM in unknown indoor environments,” Electronics, vol. 13, no. 5, p. 999, 2024, doi: 10.3390/electronics13050999.

[8] 	Z. Xu, X. Han, H. Shen, H. Jin, and K. Shimada, “NavRL: Learning safe flight in dynamic environments,” IEEE Robotics and Automation Letters, vol. 10, no. 4, pp. 3668–3675, 2025, doi: 10.1109/LRA.2025.3546069.

[9] 	A. Asgharivaskasi and N. Atanasov, “Semantic OcTree mapping and shannon mutual information computation for robot exploration,” IEEE Transactions on Robotics, vol. 39, no. 3, pp. 1910–1928, 2023, doi: 10.1109/TRO.2023.3245986.

[10] 	A. Asgharivaskasi, F. Girke, and N. Atanasov, “Riemannian optimization for active mapping with robot teams,” IEEE Transactions on Robotics, vol. 41, pp. 1077–1097, 2025, doi: 10.1109/TRO.2025.3526295.

[11] 	K. Ebadi et al., “Present and future of SLAM in extreme environments: The DARPA SubT challenge,” IEEE Transactions on Robotics, vol. 40, pp. 936–959, 2024, doi: 10.1109/TRO.2023.3323938.

[12] 	H. Azpúrua et al., “A survey on the autonomous exploration of confined subterranean spaces: Perspectives from real-world and industrial robotic deployments,” Robotics and Autonomous Systems, vol. 160, p. 104304, 2023, doi: 10.1016/j.robot.2022.104304.

[13] 	N. Botteghi, B. Sirmacek, M. Poel, C. Brune, and R. Schulte, “Curiosity-driven reinforcement learning agent for mapping unknown indoor environments,” ISPRS Annals of the Photogrammetry, Remote Sensing and Spatial Information Sciences, vol. V–1–2021, pp. 129–136, 2021, doi: 10.5194/isprs-annals-V-1-2021-129-2021.

[14] 	L. C. Garaffa, M. Basso, A. A. Konzen, and E. P. de Freitas, “Reinforcement learning for mobile robotics exploration: A survey,” IEEE Transactions on Neural Networks and Learning Systems, vol. 34, no. 8, pp. 3796–3810, 2023, doi: 10.1109/TNNLS.2021.3124466.

[15] 	M. Wang, B. Xin, M. Jing, and Y. Qu, “An exploration-enhanced search algorithm for robot indoor source searching,” IEEE Transactions on Robotics, vol. 40, pp. 4160–4178, 2024, doi: 10.1109/TRO.2024.3454572.

[16] 	E. Kaufmann, L. Bauersfeld, A. Loquercio, M. Müller, V. Koltun, and D. Scaramuzza, “Champion-level drone racing using deep reinforcement learning,” Nature, vol. 620, no. 7976, pp. 982–987, 2023, doi: 10.1038/s41586-023-06419-4.

[17] 	T. Ni, B. Eysenbach, and R. Salakhutdinov, “Recurrent model-free RL can be a strong baseline for many POMDPs,” in Proceedings of the 39th international conference on machine learning (ICML), in Proceedings of machine learning research, vol. 162. PMLR, 2022, pp. 16691–16723.

[18] 	A. Raffin, A. Hill, A. Gleave, A. Kanervisto, M. Ernestus, and N. Dormann, “Stable-Baselines3: Reliable reinforcement learning implementations,” Journal of Machine Learning Research, vol. 22, no. 268, pp. 1–8, 2021.